# Juno Skills Checklist

Mastery level can be marked as I (introduced), P (progressing), M (mastered), or as the instructor sees fit.

NOTE: The skills listed here are specific to the Juno video magnifier. Basic CCTV skills are not included on this list.

# Device Setup and Orientation

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Power on | Press the white power button for 3-5 seconds to power on the device. After boot up, you will be in the live magnifier mode. |  |  |  |
| Power off | Press and hold the power button for 10 seconds until you hear tones signaling shutdown. Juno never powers down automatically. After 5 minutes of inactivity, Juno will go into standby. Juno takes approximately 30 seconds to start up after being powered down. Juno wakes up from standby much more quickly. Briefly pressing the power button puts Juno into standby mode to save battery power. |  |  |  |
| Menus & Buttons are spoken aloud by default | Tap once to hear text and icons spoken aloud. Press Juno’s buttons to hear their functions, tap again to activate. |  |  |  |
| Restore & Hide Button Bars | Tap the Restore button bar icon in the bottom left corner of the screen. Tap again to collapse the button bar. |  |  |  |
| Previous screen | Tap the previous button icon in the bottom left corner of the menu screen to return to the previous screen. |  |  |  |
| Adjust volume and Speech Rate | Tap the Volume and Speech Rate button to display the sliders that control how loud and fast Juno speaks. These sliders control both the narration of menu information and the device voice that reads aloud after optical character recognition (OCR). Turn up the volume by pushing the left slider up toward the Volume symbol. Decrease the speed by pushing the right slider down toward the Turtle symbol. |  |  |  |
| LED Lights On and Off Toggle | There are three pairs of illumination LED lights on the device for optimal lighting. Toggle the lights off and on by tapping the Lights Off button or Lights On button. |  |  |  |
| Lines & Masks Toggle | Used to quickly locate and maintain your place on the LCD screen while in Live view. Tap the Display Reading Line button to display a reading line on your screen. Drag the line to move it. Tap the Display Reading Mask button to display a mask that blocks the screen except for a horizontal slice of the image. Drag the boundary to change the size of the masked area. |  |  |  |
| Focus Locked & Unlocked Toggle | Turn on the focus lock by tapping the Lock Focus button, which locks the focus at its current position for writing and hobby tasks. Turn off the focus lock by tapping the Unlock Focus button for optimum autofocus. |  |  |  |

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Power button | Top left. Press to power on the device. Short press to put on standby. Long press to power off. |  |  |  |
| Color mode up button | Middle left. Press to move up through color modes. Cycle through true color, grayscale, and 22 high contrast color combinations. |  |  |  |
| Color mode down button | Lower left. Press to move down through color modes. |  |  |  |
| Capture button | Top right. Press to capture a snapshot or return to live view. Optionally save a capture, recognize text, and read or listen to the text. |  |  |  |
| Zoom in button | Middle right. Increase magnification level. Press and hold for continuous zoom, press and release to incrementally zoom, or pinch and zoom on the touch screen to increase or decrease magnification. You can use one finger drag to pan around magnified images on the screen. |  |  |  |
| Zoom out button | Lower right. Decrease magnification level. |  |  |  |
| Rotating Camera | Top middle. The camera can be rotated 180 degrees so you can use it for different purposes, such as reading, handwriting or hobbies, self-viewing, spotting, and distance viewing. |  |  |  |
| Audio jack | Analog 3.5mm audio jack for headphones |  |  |  |
| USB-C | USB-C port to connect to a computer |  |  |  |
| HDMI mini | HDMI mini port to connect to a TV or monitor Plug the cable’s HDMI connector into the HDMI Mini port on the side of Juno. Plug the other HDMI connector into the HDMI port on the HDTV or monitor. Power on your HDTV or monitor and change the input source to select the device. When you connect, Juno’s screen is shown on the HDTV or monitor. You see the same thing in both places. |  |  |  |
| Charging LED | The light on the bottom left of the device indicates battery status. Begin charging Juno before first use. When the light on the left side of the device is red, the battery needs to be charged. You can use the device for about three hours once the battery is fully charged. The charging LED light will turn green when the battery is charged. The battery status symbol in the top left corner of the menu screen displays the battery level. A low battery icon will be flashed on the screen when Juno needs charging. |  |  |  |
| Collapsible stand | Manually deployed collapsible stand, on back of Juno. Opening the stand will automatically adjust the camera position into reading view. Closing the stand will put Juno into standby. Reopening the stand will wake Juno from standby. |  |  |  |

# Charging and Storage

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| To charge Juno Connect power supply cable | Insert the USB Type C cable into the USB-C slot on the left side of the device. Attach the power adapter to the USB-C cable, and plug the power adapter into an electrical outlet. You can alternately insert the cable into a computer without using the adapter, but this charging method is slower. |  |  |  |
| First use Setup | The first time you use Juno, you must follow the setup steps. You cannot exit once you start these steps and cannot use the device until you complete them. Select Date- swipe up or down in each column and tap to select the month, day, and year. Select Time- swipe up or down in each column and tap to select hours, minutes, and AM or PM Select Voice- tap to select a female or male voice to narrate as you navigate. Tap the Next button to complete the setup. The Live view is displayed. |  |  |  |
| Return to carrying case | Place the Juno into the carrying case for storage and portability. |  |  |  |

## Navigating Menus

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Navigate the button bar | Tap the Restore button bar icon in the bottom left corner of the screen. |  |  |  |
| To back out of menus and go to Live magnifier view | Tap the Previous button in the bottom left corner of the screen until you reach the live magnifier. |  |  |  |
| Open Main Menu | Tap the Main Menu icon in the top left corner of the button bar |  |  |  |
| Open File Manager | Tap the folder icon within the main menu |  |  |  |
| Change screen brightness | Tap the change screen brightness icon within the main menu. At the bottom of the screen, drag the slider switch left to dim and drag right to brighten the screen. |  |  |  |
| Open About this device | Tap the About this device icon within the main menu. Displays software version information about Juno provides technical support contact information and provides a link to the user guide. |  |  |  |
| Open Settings menu | Tap the settings icon within the main menu |  |  |  |

## Reading Settings

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Open the Reading Setting menu | Within the Settings Menu, Tap the Reading Settings icon to display the Reading Settings menu. |  |  |  |
| Change reading views | Tap the Viewing Modes icon to change the default viewing modes within the Reading Settings menu. |  |  |  |
| Toggle through viewing modes | Toggle the top right button to select the default viewing mode. Image mode shows a full screen of text without high contrast color. - Teleprompter mode shows text in paragraph form using high contrast color. - Tickertape mode shows one line of text at a time using high contrast color. |  |  |  |
| Toggle highlighting options | Tap the bottom right button to toggle the way text is emphasized as it is read aloud: Boxed Underlined |  |  |  |
| Using Zones | If you open a captured file with recognized text, then the first viewing mode is Zones. With zones, the text is separated by highlighted and numbered boxes to help you navigate around the document. Choose where you want to begin reading by tapping a zone. If you tap the Play button rather than tapping a zone, then narration starts at the beginning of the document. |  |  |  |
| Change the language | Within the Reading Settings Menus, Tap the Reading Language icon to display the Select Reading Language screen. Swipe up or down to move through the list, then select the language used for text recognition. You can only select one language at a time. Tap the Next button. Tap the name of the voice used to read documents. This voice is separate from the talking menus. Juno responds in the selected voice and the language. |  |  |  |

# Menu Settings You can control the way Juno displays and announces its menus and other screen elements through Menu Settings.

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Open Menu Settings | Within the Settings Menu, Tap the Menu Settings icon to display the Menu Settings |  |  |  |
| Toggle Talking Menus On/Off | In the Talking Menus screen, move the slider right to ON to have Juno announce buttons and menus, slide to the left to turn speech OFF. The Talking Menus slider is on by default. |  |  |  |
| Select Juno Voice | In the Talking Menus screen, Tap the Voice Selection icon to display the Select Juno Voice screen. Select the voice that Juno will use to speak the menu and button names. |  |  |  |
| Select Color Theme for Menus | In the Menu Settings screen, Tap the Color Themes icon to display the Color Theme screen. Double Tap the checkbox to select the color combinations. Use the Color Mode Up or Down buttons to cycle through preferred color contrast options for all menus. |  |  |  |
| Select Font Size for Menus | In the Menu Settings screen, Tap the Font Size icon to display the Font Size screen. The default font size is 24 points. Scroll down the list and tap your preferred font size. Swipe up to select a smaller text size or down to select a larger font. Tapping the Zoom in + button and Zoom out – buttons while in menus will also adjust text size. |  |  |  |

# User Settings Menu

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Open User Settings | Within the Settings Menu, tap the User Settings icon to display the User Settings Menu. |  |  |  |
| Choose contrast colors | Tap the Color Modes icon to display the Color Modes screen. Color combinations with checkmarks are listed double tap to check or uncheck the box to add I or remove it from the rotation. Press the Color Mode buttons to cycle through selected contrast colors in live magnifier view and on saved images. |  |  |  |
| Manage Juno Storage | Tap the Storage icon to display the Juno Storage screen. The top line shows the available storage. Try to keep more than 1 MB of available storage. |  |  |  |
| Delete All Saved Files on Juno | From the Storage screen, Tap the DELETE ALL button at the bottom of the screen. In the Delete All Data? screen, tap Yes to confirm deletion of all files on the device. |  |  |  |
| Adjust Standby Time | Tap the Standby Time icon to display the Standby Time screen. Swipe up or down and select the number of minutes that you want the system to remain idle before going into standby mode. The default is 5 minutes. |  |  |  |

# System Settings Menu

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Open System Settings | Tap the System Settings icon within the Settings menu to set the time and date and change the default file using the System Settings menu. You can also update Juno or reset the device. |  |  |  |
| Change Time and Date | Tap the Time and Date icon Tap the Edit button to the right of the date or time field to display the Select Date screen or Select Time screen. Modify the date or time by swiping up or down in each column until the correct value is selected. |  |  |  |
| Access Default Save Types | Tap the Default Save Types icon Tap the left Picture icon to select the default file type for pictures. Available file types are BMP, JPEG, PNG, and WebP. The default is JPEG. Tap the right Text icon to select the default file type for text documents. Available file types are RTF, TXT, PDF, and DOCX. |  |  |  |
| Update Juno | Update Juno with the latest software and firmware. You must download the latest Juno software onto a USB-C flash drive to perform this procedure. Juno has the most up-to-date software installed out of the box. The latest software version for Juno is 2.0.36. To check that your Juno has the most recent software version follow these steps: - Tap the RESTORE button bar icon in the bottom left corner of the screen. - Tap the MAIN MENU icon. - Tap the INFORMATION icon. - The About screen will show the App Version, OCR Version, and TTS Version as well as the Serial Number. If the latest update version listed on the APH website at aph.org (search Juno and navigate to Juno’s product page) is numerically higher than the version on the device, download the update file, which ends in .vhu. Copy the update to a USB-C flash drive. The update file needs to be placed on the root/top-level directory of the USB drive. Do not place it within any folder. Insert the USB-C flash drive with the Juno updates into Juno’s USB-C port. In the System Settings, tap the Update Juno icon. The Update Device screen is displayed. Select the .vhu file. After a moment, the UPDATE button is displayed. Tap the UPDATE button to start the update. The progress of the update is displayed in the middle of the screen. Tap OK to proceed with the update. Do not use Juno during the update process. Juno will restart at least once without any action on your part during the update. The status of the update is displayed. Note: If you restart the system during an update, the Update Device screen displays each time you restart until the update is complete. If the update completes successfully, a success message is displayed. Otherwise, a failure notification is displayed, and you must repeat this procedure. Tap OK to clear the message. |  |  |  |
| Reset to Defaults | Reset the device to the default values. All customized settings are lost, including the volume, speech rate, brightness level, and voice selection. To reset Juno’s user settings, Tap the Reset icon. The Reset All Settings? screen is displayed. Reset the user settings by tapping YES, then confirming. The settings are restored to the default values. |  |  |  |

## Single and Multi-Capture

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Capture Image | Hold Juno flat with the screen up horizontally about 8 inches over the page. Position your material so it fits within the frame. You may want to stand up to fit the image on the screen. Try turning Juno to get more text in the frame. Juno supports portrait and landscape captures. Tap on the screen to focus on the page. Be sure to zoom out to maximize the amount of text that is captured. Close the stand on Juno. Set the material you want to capture flat on a desk or table. Go to live camera view. Hold the device flat with the screen up. The camera rotates down. Hold Juno straight over the page. Frame the image until the full page is shown on the screen. Press the red Capture button, holding Juno steady until you hear a click. Tap the Restore button to display the button bars. To the right of the image are the following buttons: |  |  |  |
| Cancel | Cancel and return to the Live view |  |  |  |
| OCR | Tap the Recognize Text button to scan the document. To listen to the text from the beginning, tap the Play button. You can also tap a word or zone to start reading there. For other options, tap the Restore button |  |  |  |
| Save the image | Tap the Save button to save the image. The Add another page? screen is displayed. Tap No to save a single image. The Save Image? screen is displayed. Tap Yes to save the image then tap OK to confirm. The Record Audio Tag? screen is displayed. To record a name for the file, tap Yes. |  |  |  |
| Save an audio tag | On the Audio Tag screen, tap the Record button. Say the file name and tap the Stop button. If you want to listen to the name you recorded, tap the Play button. |  |  |  |
| Multi Capture | - When capturing a multipage document, it is much faster to scan all the pages before using text recognition. You cannot reorder the pages after they are scanned, so be sure to capture the pages in the correct order. - Position the camera, about 8 inches, over the document so the full page is framed. Press the Capture button to capture the first page. Tap the Save button. The Save? screen is displayed. Tap Add another page? to indicate that you want to create a multipage document. Tap the Next button to return to Live view and start adding pages by pressing the Capture button. When capturing multiple pages, the following buttons are displayed across the bottom of the screen: Previous, Save, Recognize Text, and Lights. Press the Capture button to return to live view to keep the image and add another. Repeat “Capture” until all pages are captured. If you are dissatisfied with a capture, tap the Previous button to discard it and take another capture. After you capture all the pages, tap the Recognize Text button. The text is displayed with Zones. You can begin reading or save the book and read it later, tap the Save button. Tap the Yes button to confirm saving and record an audio tag for the file. - To listen to the text immediately, tap the Play button. To stop, tap the Pause button. |  |  |  |
| Navigating in Multipage Documents | To quickly move around a multipage document, tap Pages at the top of the screen. In the menu that is displayed, you can tap a page number in the scrolling list or tap one of the following icons: |  |  |  |
| Start of Document | Displays the first page of the document. |  |  |  |
| Previous Page | Previous Page displays the previous page in the sequence of scanned pages. |  |  |  |
| Furthest View | Furthest Viewed displays the furthest location that you have previously viewed. If you have a 10-page document and have viewed the first five pages, for example, then you would jump to page 5. |  |  |  |
| Next Page | Next Page displays the next page in the sequence of scanned pages. |  |  |  |

## File Manager

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Access File Manager | Tap the File Manager icon, the Files screen lists the files saved on Juno. Files are sorted by the date captured or last modified, left to right, top to bottom. |  |  |  |
| View Saved Files | Tap the icon of the file you want to view or listen to its audio tag. A screenshot of the file is displayed with its name at the top of the File Information screen. |  |  |  |
| Open Document/ File | Tap the Open file button. The file is first displayed in the Zone view and the button bars are hidden. |  |  |  |
| Record audio tag | After you save or open a file, tap the Record icon. Clearly speak the file name. Think of a descriptive name for the file that will help you remember its contents later. Tap the Stop icon. The tag is saved when you tap the Previous button. Listen to your recording by tapping the Play icon. |  |  |  |
| Export Files | From the File Manager tap the icon of the file you want to export. The File Information screen is displayed. Tap the Export button. The Export To screen is displayed. Select a drive. Tap the folder (directory) on the USB-C flash drive where you want to export the file. Click the Next button. A confirmation message is displayed. Tap YES, then detach the USB-C flash drive from Juno and attach it to a USB port on your computer. If the port is not Type C, you will need a USB-A adapter. (See APH’s Juno product page for USB recommendations.) Use File Explorer to copy the file from the USB-C drive to your computer. The Transfer button, used to import and export files, is not displayed until you insert a USB-C flash drive. |  |  |  |
| Import Files | Place the files(s) you want to import in a folder on your computer. If you have homework to import, for example, then put your homework files in one folder. Copy the folder of files you want to import onto a USB-C flash drive. Plug the USB-C flash drive into Juno’s USB port. Tap the Restore button, then tap Main Menu, tap the File Manager icon to display the Files screen. Tap the Import/Export button in the lower right corner. The Select Transfer Mode screen is displayed. Tap the Import button. The Import From screen displays. Select a drive. Select the folder that holds the files that you want to import, then tap the Next button. A confirmation message is displayed. Tap Yes. The files in the folder are imported and available from the Files screen. |  |  |  |
| Copying Files to and from Juno onto a Computer | Attach the USB-Cable to ports on Juno and on your computer. Open File Explorer, where Juno is shown as a media device. - Go to /Juno/Internal Shared Storage/DCIM/Juno/. Copy the files in the three folders (subdirectories) and paste them onto your computer: - The Documents folder contains TXT, RTF, PDF, and DOCX files. - The Images folder contains BMP, JPEG, PNG, and WebP files. - JunoDoc folder contains VCB (multipage book) files. The files that comprise the VCB are also in the Documents and Images folders. You can back up these Vispero compressed book files on a computer, but no PC or web applications read this file type. Copying Files from the Computer onto Juno - Open File Explorer on your computer and copy the files you want to place on Juno. Attach the USB-Cable to Juno and your computer. - Juno is shown as a media device in File Explorer. Go to Juno/Internal Shared Storage/DCIM/Juno/ and paste the copied files into the proper folder (subdirectory): - Place TXT, RTF, PDF, and DOCX files in the Documents folder. - Place BMP, JPEG, PNG, and WebP files in the Images folder. The copied files are now available from the Files screen on Juno. Note: If the files are not copied to the correct folder, then they will not display on the Files screen. |  |  |  |
| Rename a Document | You can apply an audio tag to any file that you save or import. You can rename or change the audio tag for a document at any time, by following the steps to record an audio tag. However, a file exported to a USB-C flash drive or copied to a computer will not keep its audio tag, so the audio tag is only useful on Juno. When copying files from the computer onto Juno, the file names created on the computer are carried over to the File Manager on Juno making it easier to locate documents. |  |  |  |
| Delete a Document | From the File Manager screen Tap the icon representing the file you want to delete. Tap the Edit button in the bottom right corner. The Edit screen is displayed. Tap the Delete button in the bottom right corner. After you confirm the deletion, the selected file is removed, and the updated list of files is displayed. |  |  |  |
| Editing Files | Tapping the Edit button. On the Editing screen, you have the following buttons for images (BMP, JPEG, PNG, and WebP files) |  |  |  |
| Flip image | Mirror the image |  |  |  |
| Rotate forward | Rotate the image clockwise 90 degrees |  |  |  |
| Rotate back | Rotate image counterclockwise 90 degrees |  |  |  |

## Reading, Handwriting & Hobby, Distance View, and Self-View

| Skill | Action/Gesture | Image | Mastery Level | Date |
| --- | --- | --- | --- | --- |
| Text recognition | The best way to freeze or capture text is to close the stand and hold the device flat with the screen up. The camera automatically rotates 90 degrees so it is pointing down. Hold Juno over a document to get the best page magnification for magazines, catalogs, menus, photographs, and other printed material. To capture an 8.5-inch by 11-inch page, lift Juno approximately 8 inches above the document. |  |  |  |
| Handwriting/ Hobby | You can use Juno to write by opening the stand and rotating the camera so it is pointed down about halfway. You place paper behind the device, then write on it while watching yourself write on the screen. If you enable Focus Lock, the camera focuses on the paper instead of your hand. You can work on hobbies that require fine motor skills, such as coloring/drawing, painting/trimming nails, and knitting, in the same manner. |  |  |  |
| Distance View | To view something at a distance, such as a whiteboard, open the stand and rotate the camera so that it is parallel with the stand. Can view objects at a distance up to 15 ft. away. |  |  |  |
| Self-view | To view yourself, position the device so you are looking at the screen. The stand can be open or closed. Rotate the camera until it faces you and you can see yourself on Juno’s screen or on an attached display device. When the camera is facing you, the image is flipped so it is like you are looking in a mirror. A capture of your image, however, is not flipped. LED lights are off when you enter this mode, but you can turn them back on by toggling the LED lights on/off in the button bar menu. |  |  |  |
| Spotting | With the reading stand closed, point the camera at what you want to view. Press the yellow Zoom In or Zoom Out buttons to increase or decrease magnification. To take a snapshot of the image on the screen, press the Capture button, holding Juno steady until you hear a click. This is ideal for viewing labels, price tags, receipts, posters, business cards, and more. Press the Capture button again to return to the Live view. |  |  |  |